IKIRERE ORBITAL LABS AFRICA

OPEN PROBLEM

Real-Time Many-to-Many Conjunction Screening at Orbital Scale

Posed by Ikirere Orbital Labs Africa

2026  ·  research@ikirere.com  ·  ikirere.com

# The Problem

Given a catalog of 15,000+ actively tracked orbital objects, compute all pairwise conjunction risks - including contextual orbital risk weighting - at operational real-time constraints on commodity CPU hardware.

# Why It Matters

## The Kessler Cascade

In 1978 Donald Kessler and Burton Cour-Palais described a cascade failure scenario that has since become the defining existential risk of orbital infrastructure. A collision in Low Earth Orbit generates debris. Each fragment strikes other satellites. More debris, more collisions - until the orbital shell becomes self-sustaining in its own destruction.

This is not theoretical. The 2009 Iridium-Cosmos collision and the 2021 Cosmos-1408 ASAT test each generated thousands of fragments still in orbit today. The LEO shell between 400 and 600 km is approaching critical density.

## The Scale of the Problem

The pair count is not fixed. It grows with the catalog, which grows continuously:

| Catalog | Raw Pairs | Est. Year |
| --- | --- | --- |
| 15,447 active objects (today) | 119,267,631 pairs | 2026 |
| 27,000 all tracked objects | 364,486,500 pairs | 2026 |
| 50,000+ (megaconstellations + growth) | 1,249,975,000 pairs | ~2030 |

SpaceX has regulatory approval for 42,000 Starlinks alone. A solution that works today must be designed to scale to the 2030 catalog.

## Why Current Systems Are Insufficient

| System | Performance |
| --- | --- |
| Space-Track (USSPACECOM) | 8-hour screening cadence |
| LeoLabs | Under 30 seconds (proprietary, undisclosed) |
| GPU-accelerated propagation (Cambridge, 2026) | 4ms propagation only - not the full pipeline |
| Full pipeline on CPU at operational real-time constraints | Publicly available implementations remain computationally constrained at constellation scale |

GPU-accelerated propagation has been demonstrated at scale. The full pipeline on commodity CPU - pairwise geometry, time of closest approach computation, and contextual risk scoring - remains computationally constrained at constellation scale.

Why CPU matters: A CPU solution runs on any ground station, any laptop, any operator anywhere in the world. GPU requires expensive specialised hardware unavailable to most satellite operators, university labs, and researchers in emerging markets. CPU is the democratising result.

# What a Solution Must Do

For a catalog of n orbital objects, a valid solution must:

- Screen all pairs where altitude difference is below a configurable threshold (eliminating geometrically impossible conjunctions)

- Compute the time of closest approach (TCA) and minimum separation for surviving pairs

- Produce a risk score per pair that accounts for: geometric proximity, relative velocity, time urgency, positional uncertainty, and orbital shell population density

- Return results ranked by risk, highest first

- Complete the full pipeline within operational real-time constraints on a modern CPU

- Scale efficiently as orbital catalog density increases over time

Accuracy requirements:

- TCA accurate to within 2 seconds

- Miss distance accurate to within 1 km for LEO objects

- All pairs within the altitude threshold must appear in the output - no false negatives

# The Gap to Close

Existing CPU implementations process the full catalog serially. The constraint is not the algorithm - it is the architecture. The gap between current state and operational real-time constraints is large.

Three distinct computational problems must be solved simultaneously in a single pass, without inter-stage data transfers that would dominate the latency budget:

- Propagation for N satellites across M future time steps

- Pairwise geometry for all surviving pairs (up to millions) simultaneously

- Per-pair risk scoring incorporating a whole-catalog density metric

# If You Are Working on This

We are looking for researchers, engineers, and collaborators who are working on - or interested in - this class of problem. If you have solved a component, have relevant work, or want to collaborate, reach out.

| Organisation | Ikirere Orbital Labs Africa |
| --- | --- |
| Email | research@ikirere.com |
| Website | ikirere.com |

Software first.  Hardware second.  Space third.